
200W  laser spot welding machine
The laser spot welding machine dedicated to gold and silver jewelry, golf, fill holes for electronic components, welding trachoma, weld inlay mouth and so on. Welding firm, beautiful, non-distortion, simple operation, easy to use.
Profile
  200W laser spot welding machine is used for perforation and spot welding of sand hole for gold and silver jewelries. Laser spot welding is an important application of laser materials processing technology. Spot welding is thermal conduction, namely, the laser radiates the surface of the part, and the heat on the surface expands inside through heat conduction. By controlling
parameters such as laser pulse width, energy, peak value and repeating frequency, the part will melt, and thus forming specific molten pools. Due to its unique benefit, the product has been successfully applied to gold and silver jewelries processing, and welding of small-sized parts. 
Characteristics
● Energy, pulse width, frequency and facular can be adjusted within a wide range to achieve different welding effects. The parameters are adjusted by the control rod in the sealed cavity, simple and efficient.
● Ceramic focusing cavity imported from the U.K. is employed, corrosion resistant, high temperature resistant, efficient in optical/electrical conversion. The focusing cavity's service life is 8 to 10 years, and the service life of the xenon lamp is over 8 million times.
● World-leading automatic light shielding system is employed to remove stimulation of light on eyes during operation.
●with out maintenance for working 10000 hours
● Personalized design in compliance with ergonomics principles.
Advantages
Fast, efficient, deep, little distortion, little affect area, quality welding, welding points free from pollution, and energy saves.

 Technical Parameters
	 Maximum average power
	200W

	Laser wavelength
	1064nm

	Max Pulse energy
	120J

	Laser welding depth
	0.1-3.3mm

	The pulse width
	0.3-20ms

	Batter, laser welding frequency
	0.5-20Hz

	Adjustable range of spot size
	0.2-2mm

	Control system
	 Industrial computer

	Machine Power consumption
	          ≤4kw

	The demand for electricity
	220V±0%/50Hz/30A

	System components and environmental system components
	Host computer, Cold water tank

	Supplies
	Xenon lamp, filter, protection lens, argon gas

	Installation environment
	Installation Location: 4 square meters.

	Environment
	clean, no dust or less dust

	Temperature
	55℉(13℃)to 82℉(28℃)

	Humidity
	5% to 75%non-condensing.

	Power
	220V50HZ Alternating current,.30A an air switch, voltage stability


 

Sample Pictures
[image: image1.jpg]


 [image: image2.jpg]


 [image: image3.jpg]




 INCLUDEPICTURE "http://www.szsnlaser.com/upload/0050626526.jpg" \* MERGEFORMATINET [image: image4.jpg]Q%



   [image: image5.jpg]


   [image: image6.jpg]


   [image: image7.jpg]{5
Fgtiabnib,
ruv‘«':‘






 INCLUDEPICTURE "http://www.szsnlaser.com/upload/36318037.jpg" \* MERGEFORMATINET [image: image8.jpg]




 INCLUDEPICTURE "http://www.szsnlaser.com/upload/76468784.jpg" \* MERGEFORMATINET [image: image9.jpg]




 INCLUDEPICTURE "http://www.szsnlaser.com/upload/1928624173.jpg" \* MERGEFORMATINET [image: image10.jpg]




 INCLUDEPICTURE "http://www.szsnlaser.com/upload/e0a6523.jpg" \* MERGEFORMATINET [image: image11.jpg]




